Hkkjr lsok laLFkku

y[kuÅ

fufonk la0&01@Hkk0ls0la0@2016&17

v/kksgLrk{kjh }kjk pUnzHkkuq xqIr xzke fodkl ;kstuk  o vkW[k rFkk tujy vLirky] gFkkSa/kk dksVok lM+d dh fcfYMax fjis;fjax o isafVx rFkk ckm.MªhokWy cukus gsrq fuEufyf[kr dk;Z gsrq VsDuhdy o izkbl fcM dh vyx&vyx eksgjcan fufonk,a fu;e o “krksZa lfgr fnukad 31-08-2016  lka;dky 16%30 cts rd vkeaf=r dh tkrh gSaA  (A) +( B)

(A)

Repairing/ Painting & Distempering of old Boundary Wall, Hospital Building, Training Centre, Hostel Building, Pump House & Garrage of Chandra Bhan Gupta Eye Hospital & Training Center Hathaudha  Kotwa Sadak (Barabanki)

| Sl.no. | Particulars | Qty. | Unit |
| --- | --- | --- | --- |
| 1111 0 | Providing and applying plaster of paris putty one priming coat & two coat oil distemper or exterior paint of best quality on new work including supply of all materials, labour and T &P etc . required for proper completion of the work. | 1438.00 | Sqm. |
| 2 | Finishing walls with one coat of wall putty and two coats of water proof cement or Exterior paint of approved quality on previously old treated surface with this paint to give an even shade including supply of all material labour and T & P ect required for proper completion of the work. | 6361.00 | Sqm. |
| 3 | Painting or vanishing on iron works in small areas or new wood with one coat of ready mixed priming paint & one coat of approved superior quality ready mixed paint of clear synthetic varnish including supply of all materials, labour and T & P etc. required for proper completion of work. Paint to be used should confirm to the I.S.I No. 103 1950. Details of cost for an area for 650 sft (Priming coat on wood work.) | 630.00 | Sqm |
| 4 | Misc repairing work such as plaster/window glass fitting, repairing of staircase etc. | L.S |  |

(B)

Construction of Boundary wall of Chandra Bhan Gupta Eye hospital & Training Centre Hathaudha Kotwa Sadak (Barabanki) as per following description 5013.88 Sqft.

| Sl.no. | Particulars | Qty. | Unit |
| --- | --- | --- | --- |
| 1 | Earth work in Excavation in trenches for foundation pips cable etc in ordinary soil (Loan clay or sand) including lift upto 1.5 m. lead upto 50 m and dressing of sides and ramming of bottom and disposal of surplus exdavated earth as directed by the Engineer i/c within a lead of 50m. | 23.00 | Cum. |
| 2 | Providing and laying Cement concrete 1:6:12(1 cement: 6 fine sand :12 graded brick aggregate 40, mm. nominal size) including supply of all materials! labour and T &P etc. required of proper completion of the work and curing complete, also including cost of formwork in foundation & floors. | 6.00 | Cum |
| 3 | First Class brick work in 1:6 cement and fine sand of 1.25 F.M. mortar in foundation and plinth including supply 6f all materials, labour and T&P etc. required for proper completion of the work. | 18.50 | Cum |
| 4 | DPC 2.50 cm. (1”) thick and of cement concrete consisting of one part cement 11/2 parts approved coarse sand and 3 parts approved 13 mm (11/2) graded stone grit and including water proofing materials as ordered by the Engineer-in-charge in the proportion as specified by the manufacturers. | 15.00 | Sqm. |
| 5 | First Class brick work in 1:4 cement and coarse sand of 1.25 F.M. mortar but in the super-structure including necessary cutting and moulding of bricks as required and also including honey comb brick, work thickness of wall not to be less than 114 brick. Add for extra labour to above items. | 14.00 | Cum |
| 6 | Mild steel or iron in, plain work such as reinforced concrete or reinforced brick work (when not included in over all rates) wrought to required shape as necessary including bending for proper completion of the work and including supply of all steel and wastage & over lapping & hooks. | 105.34 | Kg. |
| 7 | 12 mm thick plaster in 1:5 cement & coarse sand & fine sand including cost of all materials labour T&P etc. required for proper completion or work. | 231.00 | Sqm. |
| 8 | Finishing walls with water proof cement wall putty and Exterior paint of approved make and quality on new work with two coats to give an even shade. Including supply of all material labour and T&P etc required for proper completion of the work. | 231.00 | Sqm. |
| 9 | Filling of sand/earth in plinth including supply of all materials, labour, T&P etc. and also including cartage of earth with distance of 8 km. from the site & also including watering , raming & dressing the surface to required level etc. required for proper completion of the work. | 211.00 | Cum |
| 10 | Dismantling brick work or stone work in lime or cement mortar and including stacking of materials as directed by Engineer-in-charge with in 60 meter’s distance (Sl.no. 692) | 4.20 | Cum |
| 11 | Ckkm.MªhokWy fuEu lkbt esa uD”ks ds vuqlkj cukuk gksxkA ewy uD”kk dk;kZf/kdkjh] xzke fodkl ;kstuk gFkkSa/kk ds ikl miyC/k jgsxkA |  |  |

74’8’’

68’7’’	65’9’’

75’3’’

fufonknkrk mijksDr fcfYMax dk fujh{k.k dHkh Hkh fufonk Hkjus ls iwoZ dj ldrs gSaA  fufonk,a fnukad 31-08-2016 ds 12%00 cts rd ;k blls igys v/kksgLrk{kjh ds dk;kZy; ls “kqYd #0 100-00 ¼#0 ,d lkS ek=½ ¼vizfrns;½ tek dj izkIr fd;k tk ldrk gSA  fufonk,a fnukad   31-08-2016 dks lka;dky 5%00 cts bPNqd fufonknkrkvksa vFkok muds vf/kd`r izfrfuf/k;ksa ds le{k v/kksgLrk{kjh vFkok muds }kjk euksuhr izfrfuf/k }kjk izkIr@[kksyh tk,xhA  ;fn fufonk [kqyus dh frfFk dks vodk”k gksrk gS rks fufonk,a vxys dk;Z fnol dks fu;r le; ij izkIr@[kksyh tk,xhA  fufonk gsrq vusZLV euh #0 20]000-00 ¼#0 chl gtkj ek=½ Mh0Mh0@,Q0Mh0vkj0 ds :Ik esa Hkkjr lsok laLFkku] y[kuÅ  ds i{k esa ns; gksxhA  fufonknkrk dks vko”;drkuqlkj vk;dj izek.k&i=] /kjksgj jkf”k] vuqHko izek.k &i= fufonk ds izFke Hkkx esa izLrqr djus gksaxs rFkk fufonk ds f}rh; Hkkx esa njsa fu;e ,oa “krsZa bR;kfn gksxhA  fufonknkrk vius izLrko egklfpo] Hkkjr lsok laLFkku] 2&jk.kk izrki ekxZ] eksrhegy] y[kuÅ  fLFkr dk;kZy; esa fnukad 31-08-2016 dks lka;dky 16%30 cts rd tek dj ldrs gSaA

egkea=h

Hkkjr lsok laLFkku

2&jk.kk izrki ekxZ]

eksrhegy] y[kuÅ&226001

izfrfyfi uksVl cksMZ]z] xzke fodkl ;kstuk o tufgrdkjh vLirky gFkkSa/kk] ckjkcadhA